NULA

Carbon Factor Master Library

Controlled factor register, provenance rules and production-status classification

Version 1.1 — Revised Draft for Independent Review
23 August 2026

Scientific status: scientifically grounded factor register aligned to Nula Carbon Calculation Methodology Statement v1.1; revised following technical/LCA review; not independently validated, assured or certified.

# Document Control

| Document owner | Nula |
| --- | --- |
| Version | 1.1 |
| Status | Revised draft for independent technical/LCA review |
| Effective date | 23 August 2026 |
| Review cycle | At least annually; on material source/method change; and before a fallback intensity exceeds its defined re-derivation limit |

Important limitation. This document defines the intended calculation and governance methodology for Nula. It does not constitute assurance, certification, a regulatory inventory, or a claim that every current production factor has been independently validated. Results must be presented as estimates unless a separately verified calculation supports a stronger claim.

# 1. Purpose

This library is the controlled register of emission factors and fallback intensities authorised for Nula calculations. A factor is not production-ready merely because it is present in this document: each row carries a source class, confidence grade and production status.

v1.1 control principle: the library records not only a central factor value but also the evidence needed to interpret it safely — method grade, uncertainty status, source/version, validity, boundary, and any special treatment such as aviation non-CO₂ effects. A factor can be numerically correct yet unsuitable for a specific transaction if those conditions are not met.

Transaction-led reality: because bank/card data rarely contain litres, kWh, passenger-kilometres or item quantities, Grade C/D/E factors are expected to be materially represented in the MVP. Nula will measure the actual grade distribution on representative Dutch transaction data rather than assume it.

# 2. Required Factor Metadata

| Field | Requirement |
| --- | --- |
| factor_id | Stable unique identifier; never reused for a materially different factor. |
| activity_or_category | Physical activity, product class or spend category. |
| factor_value | Numeric emission intensity. |
| unit | e.g. kg CO₂e/L, kg CO₂e/kWh, kg CO₂e/pkm, kg CO₂e/€reference-year. |
| boundary | WTW, TTW, WTT, cradle-to-retail, supply-chain/EEIO, etc. |
| geography | NL, EU or other applicable scope. |
| reference_year | Year/version to which the factor applies. |
| source_name + URL | Human-readable provenance and source URL. |
| methodology_type | authoritative_activity, product_LCA, detailed_spend, derived_spend_fallback, generic_fallback, placeholder_dev. |
| confidence_grade | A, B, C, D, E or X. |
| is_active | Whether the factor is eligible for new calculations. |
| valid_from / valid_to | Controlled effective dates. |
| notes | Important assumptions, caveats or source-specific guidance. |
| source_version_or_publication | Source publication/version/date sufficient to reproduce the factor. |
| uncertainty_status | Quantified interval/range where supported; otherwise 'not quantified'. Never invent a numeric band. |
| special_treatment | Material methodological treatment, e.g. aviation non-CO₂ uplift or electricity contractual basis. |
| rederive_by | Latest date/year for mandatory suitability review or re-derivation of Nula-derived spend factors. |
| supersedes / superseded_by | Links factor lineage without overwriting historical records. |

# 3. Authoritative Dutch Activity Factors — 2026

# Aviation disclosure: the European 700–2,500 km factor is 0.172 kg CO₂e/passenger-km. The source explicitly includes non-CO₂ effects and states a mean uplift factor of 0.7 over direct CO₂. This treatment is a material part of the source factor and must travel with the calculation record; it is not a generic Nula multiplier.

# Electricity disclosure: the 0.244 kg CO₂e/kWh unknown-source factor is explicitly described by the source as grid mix / location based. Source-specific green electricity and Guarantees of Origin require separate evidence and treatment; Nula must not automatically convert a green-contract claim to zero.

| Factor ID | Activity | Value | Unit | Boundary | Geo | Year | Source | Grade | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NL-FUEL-E10-2026 | Petrol E10 | 2.794 | kg CO₂e/litre | WTW (TTW 2.139 + WTT 0.655) | NL | 2026 | CO₂-emissiefactoren.nl / CE Delft STREAM | A | Production candidate |
| NL-FUEL-DIESEL-B7-2026 | Diesel B7 | 3.251 | kg CO₂e/litre | WTW (TTW 2.462 + WTT 0.788) | NL | 2026 | CO₂-emissiefactoren.nl / CE Delft STREAM | A | Production candidate |
| NL-CAR-AVG-2026 | Average car, fuel unknown | 0.190 | kg CO₂e/vehicle-km | WTW | NL | 2026 | CO₂-emissiefactoren.nl / CE Delft STREAM | A | Production candidate when actual fuel is unavailable |
| NL-OV-AVG-2026 | Public transport average | 0.036 | kg CO₂e/passenger-km | WTW | NL | 2026 | CO₂-emissiefactoren.nl | A | Production candidate |
| NL-TRAIN-ELECTRIC-2026 | Electric train | 0.019 | kg CO₂e/passenger-km | WTW (WTT 0.019) | NL | 2026 | CO₂-emissiefactoren.nl / CE Delft STREAM | A | Production candidate; changed materially from earlier years |
| NL-FLIGHT-EU-2026 | Flight, European 700–2,500 km | 0.172 | kg CO₂e/passenger-km | WTW; published value includes aviation non-CO₂ climate effects. Source states a mean uplift factor of 0.7 over direct CO₂; published WTW 0.172, TTW 0.152, WTT 0.021 kg CO₂e/pkm. | NL methodology | 2026 listing | CO₂-emissiefactoren.nl; Milieu Centraal 2022; CE Delft ANCO 2023 | A/B | Conditional production candidate; route distance required; disclose non-CO₂ treatment; do not imply comparability where another calculator uses different aviation treatment |
| NL-ELEC-GRID-2026 | Electricity, source unknown/grid mix | 0.244 | kg CO₂e/kWh | Unknown-source grid mix / location-based factor; production + upstream energy chain; infrastructure separately reported by source | NL | 2026 | CO₂-emissiefactoren.nl | A | Production candidate only when source cannot be established; not a market-based contractual electricity result |
| NL-ELEC-GREY-2026 | Grey electricity | 0.483 | kg CO₂e/kWh | Production + upstream energy chain | NL | 2026 | CO₂-emissiefactoren.nl | A | Production candidate when grey electricity applies; supplier electricity-label data require appropriate upstream-chain treatment |
| NL-GAS-G-2026 | Natural gas (G-gas) | 2.134 | kg CO₂e/Nm³ | WTW (TTW 1.779 + WTT 0.355) | NL | 2026 | CO₂-emissiefactoren.nl | A | Production candidate |

Application rule: prefer actual physical fuel/electricity use over generic vehicle-distance or spend proxies when those physical data are available. The Dutch transport source explicitly states that actual fuel/electricity use is more accurate. [1]

# 4. Product/LCA Data Sources

# Use restriction: where a source has not undergone the critical review required for comparative assertions, its product-level values may support estimation within stated limitations but must not be used by Nula to make or imply unsupported head-to-head environmental superiority rankings.

| Library entry | Coverage | Factor format | Source | Geo | Grade | Status / condition |
| --- | --- | --- | --- | --- | --- | --- |
| RIVM-FOOD-2024 | Generic food products (~400) | Product-specific kg CO₂e per functional unit as provided in source | RIVM Database Milieubelasting Voedingsmiddelen 2024 | NL consumption context | B | Eligible after item mapping; preserve RIVM disclaimer |
| EU-PEF-APPAREL | Apparel & footwear methodology | No universal fixed factor | EU PEF / 2025 apparel-footwear PEFCR | EU | B | Methodological framework; requires suitable lifecycle inventory/product data |
| EU-PEF-GENERAL | Product Environmental Footprint methodology | No universal fixed factor | European Commission Environmental Footprint Recommendation | EU | B | Methodological framework |

# 5. Nula-derived Broad Spend Fallbacks — Reference Year 2020

The following ratios are calculated by Nula from CBS 2020 average household footprint divided by CBS 2020 average household expenditure. They are screening fallbacks, not official CBS-published emission coefficients. They require price-year adjustment before application to another nominal year. [2][3]

v1.1 staleness control: CPI adjustment does not refresh the underlying physical carbon intensity. Nula-derived broad ratios must be formally reviewed at least annually and re-derived when materially newer compatible data become available. Under Methodology v1.1, a ratio whose reference dataset is more than five calendar years older than the transaction year cannot be used as a normal production factor without documented reviewer approval; otherwise it is downgraded to Grade E or withheld.

Consequently, for 2026 transactions the 2020 broad ratios below are retained for lineage, testing and screening, but are not normal Grade-D production factors. They are marked Grade E / re-derivation required pending a newer compatible derivation or documented reviewer approval.

| Factor ID | Category | Value | Unit | Derivation | Grade | Status |
| --- | --- | --- | --- | --- | --- | --- |
| NL-SPEND-FOOD-BROAD-2020 | Food & non-alcoholic beverages | 0.667 | kg CO₂e/€2020 | CBS footprint 2,972 kg ÷ expenditure €4,458 | E | 2026: re-derivation required; lineage/screening only unless documented reviewer approval |
| NL-SPEND-TRANSPORT-BROAD-2020 | Transport | 0.714 | kg CO₂e/€2020 | 2,824 ÷ 3,955 | E | 2026: re-derivation required; lineage/screening only unless documented reviewer approval |
| NL-SPEND-FURNISH-BROAD-2020 | Furnishings & household appliances | 0.292 | kg CO₂e/€2020 | 667 ÷ 2,282 | E | 2026: re-derivation required; lineage/screening only unless documented reviewer approval |
| NL-SPEND-HORECA-BROAD-2020 | Restaurants & hotels | 0.311 | kg CO₂e/€2020 | 513 ÷ 1,651 | E | 2026: re-derivation required; lineage/screening only unless documented reviewer approval |
| NL-SPEND-CLOTHING-BROAD-2020 | Clothing & footwear | 0.222 | kg CO₂e/€2020 | 310 ÷ 1,394 | E | 2026: re-derivation required; lineage/screening only unless documented reviewer approval |
| NL-SPEND-RECREATION-BROAD-2020 | Recreation & culture | 0.231 | kg CO₂e/€2020 | 699 ÷ 3,020 | E | 2026: re-derivation required; lineage/screening only unless documented reviewer approval |

# 6. Price-Year Adjustment Rule for Spend Factors

Adjusted spend = Current spend × CPI(reference year, category) / CPI(transaction period, category)

Estimated CO₂e = Adjusted spend × Spend intensity(reference year)

Use the most specific comparable CBS CPI/ECOICOPv2 series available. CBS reports that the 2025=100 series recodes 2010–2024 articles to ECOICOPv2 where possible. If only a broader comparable index is available, use it with reduced confidence; if no valid comparable index exists, do not manufacture a precise adjustment. [4]

CPI changes the monetary reference basis only. It does not account for structural changes in grid mix, technology, fuel efficiency, supply chains or category composition. A CPI-adjusted old intensity therefore must not be represented as a newly observed carbon intensity.

For each spend factor, store the CPI series identifier, reference period, transaction-period index, adjustment ratio and resulting adjusted spend so the calculation can be reproduced.

# 7. Factor Selection Rules

# Anti-double-counting: each underlying purchase/expenditure amount contributes once. When activity/product evidence replaces a spend estimate, exclude that transaction from any overlapping residual spend pool.

# Residual-spend aggregation: apply broad/detailed spend factors only to expenditure not already represented by an activity or product/LCA calculation. Retain transaction ID, selected factor ID, superseded method (if any) and residual-allocation status.

Match geography and reference year first.

Prefer physical activity over expenditure when credible activity is available.

Prefer product/item lifecycle data over broad spend factors when the item is reliably identified.

Do not infer physical quantities (litres, km, kWh) solely from merchant identity or price unless the inference method and price data are separately documented and confidence is reduced.

Do not assign one product factor to a multi-category marketplace.

Use only one active factor for a given controlled factor version/key in a calculation context.

Retain historical factor IDs and results even after factors are superseded.

# 8. Production Status and Gaps

| Domain | Status | Requirement / limitation |
| --- | --- | --- |
| Transport fuels | Candidate ready | Authoritative 2026 physical factors available |
| Road/rail/public transport | Candidate ready | Physical km/pkm route available; generic factors available |
| Home electricity/gas | Candidate ready | Physical kWh/Nm³ preferred |
| Food, item-level | Conditional | RIVM library available; item matching and boundary selection required |
| Food, spend-only | Fallback — re-derivation required for 2026 | 2020 broad ratio retained for lineage/screening; newer compatible derivation needed |
| Clothing | Conditional/fallback | EU methodology available; product inventory/factors incomplete |
| Electronics | Gap/fallback | Product-level library not yet controlled; detailed EEIO/spend mapping needed |
| Services | Gap/fallback | Detailed EEIO/COICOP factor library needed |
| Unknown/other | Low-confidence only | Ask user where practical; otherwise explicit low-confidence fallback with uncertainty status |
| Aviation non-CO₂ | Defined / disclosure required | Store uplift treatment, source and version; benchmark only against boundary-compatible references |
| Electricity contractual basis | Partial | Grid-mix/location-based path defined; market-based/GvO path requires explicit evidence rules |
| Quantitative uncertainty | Architecture required | Store supported interval/range or 'not quantified'; grade is not a numeric error band |
| Grade distribution calibration | Pending | Measure A–E/X distribution on representative Dutch transaction sample before typical-quality claims |

# 9. Change Control

New source year or materially different methodology creates a new factor version/ID.

Changes must be logged with effective date, reason and validation impact.

No factor can move to production status without provenance review and at least one regression/validation case.

Annual review should compare the current Dutch factor list with prior year values and assess whether assumptions remain valid.

Independent benchmarking should be recorded for selected high-impact factors/cases where a credible external reference exists. Acceptance is based on explainable differences in boundaries, geography, year, allocation and special treatments — not forced numerical equality.

Historical factor versions referenced by transactions must remain retrievable to an authorised audit process after supersession. Ordinary users must not be able to modify controlled factor records.

Retrospective recalculation is exceptional and must be explicitly documented.

# 10. Revision Note — v1.1

Version 1.1 aligns the Master Library with Methodology Statement v1.1. It adds explicit aviation non-CO₂ treatment, electricity accounting disclosure, anti-double-counting/residual-spend controls, factor staleness and re-derivation rules, uncertainty metadata, product-comparison restrictions, historical reproducibility, grade-distribution calibration and external benchmarking controls. The 2020 broad spend ratios are retained for lineage but downgraded for 2026 use because their reference dataset is now beyond Nula's five-year governance threshold.

Source verification note (23 August 2026): CO₂-emissiefactoren.nl describes unknown-source electricity as grid mix / location based and gives 0.244 kg CO₂e/kWh for 2026; its European-flight page gives 0.172 kg CO₂e/passenger-km and explicitly states that non-CO₂ effects are included using a mean uplift factor of 0.7 over direct CO₂.

# References

[1] CO₂-emissiefactoren.nl, Personenvervoer 2026. https://co2emissiefactoren.nl/factoren/2026/13/personenvervoer/

[2] CBS (2026), “Verschillen in de broeikasgasvoetafdruk van Nederlandse huishoudens.” https://www.cbs.nl/nl-nl/longread/rapportages/2026/verschillen-in-de-broeikasgasvoetafdruk-van-nederlandse-huishoudens?onepage=true

[3] CBS StatLine, “Bestedingen van huishoudens; bestedingscategorieën” (2020 expenditure data). https://www.cbs.nl/nl-nl/cijfers/detail/83676NED

[4] CBS (2026), “Consumer price index from 2026” / ECOICOPv2 recoding and 2025=100 base. https://www.cbs.nl/en-gb/longread/rapportages/2026/consumer-price-index-from-2026?onepage=true

[5] RIVM, “The environmental impact of food products,” Database Milieubelasting Voedingsmiddelen 2024, published 30 March 2026. https://www.rivm.nl/en/food-and-nutrition/sustainable-food/environmental-impact-of-food-products

[6] European Commission (2021), Recommendation on the use of Environmental Footprint methods. https://environment.ec.europa.eu/publications/recommendation-use-environmental-footprint-methods_en

[7] European Commission (2025), “New EU rules for measuring environmental impact of clothes and shoes.” https://environment.ec.europa.eu/news/new-eu-rules-measuring-environmental-impact-clothes-and-shoes-2025-06-25_en

[8] CO₂-emissiefactoren.nl, Downloads and change history, 2015–2026. https://co2emissiefactoren.nl/downloads/

[9] CO₂-emissiefactoren.nl, Benzine E10 2026. https://co2emissiefactoren.nl/factoren/2026/30/66/brandstoffen-voertuigen-en-schepen-fossiele-brandstoffen-met-bio-bijmenging-benzine-e10/

[10] CO₂-emissiefactoren.nl, Diesel B7 2026. https://co2emissiefactoren.nl/factoren/2026/30/67/brandstoffen-voertuigen-en-schepen-fossiele-brandstoffen-met-bio-bijmenging-diesel-b7/

[11] CO₂-emissiefactoren.nl, Electric train 2026. https://co2emissiefactoren.nl/factoren/2026/53/196/personenvervoer-openbaar-vervoer-trein-elektrisch/

[12] CO₂-emissiefactoren.nl, European flight 700–2,500 km. https://co2emissiefactoren.nl/factoren/2026/51/189/personenvervoer-vliegtuig-europees-700-2500-km/

[13] CO₂-emissiefactoren.nl, Electricity 2026 / unknown source grid mix. https://co2emissiefactoren.nl/factoren/2026/11/elektriciteit/?unit=kwh

[14] CO₂-emissiefactoren.nl, Energy-generation fuels 2026 / natural gas G-gas. https://co2emissiefactoren.nl/factoren/2026/10/brandstoffen-energieopwekking/